NATIONAL UNIVESITY OF SCIENCE AND TECHNOLOGY

APPLIED PHYSICS DEPARTMENT

SPH 1203– THERMAL PHYSICS

BSc HONOURS APPLIED PHYSICS: PART I:

MAY  2013					DURATION: 3HOURS

ANSWER ALL PARTS OF QUESTION ONE IN SECTION A AND ANY THREE QUESTIONS FROM SECTION B. SECTION A CARRIES 40 MARKS AND SECTION B CARRIES 60 MARKS.

SHOW ALL YOUR STEPS CLEARLY IN ANY

SECTION A

1.	(a)  (i) Explain, briefly the differences between:

Heat Conduction and Heat Convection.  Define the quantities involved in

the laws of each.                                                                                    [6]

(ii) State Stefan Boltzmann Law defining the quantities involved.                    [4]

(b)	A gas with an initial volume of 0.30m3 exerts a pressure

p=2x105Nm-2. At this pressure, it expands to a final volume of               0.45m3. Find the work done by the gas.                                                        [6]

(c)	A liquid is irregularly stirred in a well-insulated container and thereby

undergoes a rise in temperature. If we regard the liquid as the system

- Has heat been transferred?

- Has work been done?

(ii)      What is the sign of ∆U?                                                                         [6]

(d)	Show that  Cp-Cv=R	                                                                         [4]

(e)	Show that in an adiabatic process this equation holds TVγ-1=A where A is a constant.	                                                                                            [4]

(f)	(i)  Given that an hydrostatic system has the thermodynamic coordinates P, V, T show by defining the thermodynamic quantities involved that:

.dp= βpkTdT	  where  βp  and kT have their usual meaning.                       [6]

SECTION B

2.	(a)	Explain the first law of thermodynamics                                                             [4]

(b )    (i)      Draw a schematic diagram of an elementary refrigirator showing

the role of each section in the cycle.                                                      [6]

(ii)          Show the cycle in a   p – V  diagram that includes arrows that

indicate the direction of the process and that of the heat flow.            [6]

- From (ii) derive an expression for the efficiency of the cycle.            [4]

3. (a) Make a comparison  of a reversible and an irreversible processes.                               [4]

(b) Show that the efficiency of a thermal engine operating according to a reversible Carnot cycle is independent of the working substance and depends only on the two

temperatures.                                                                                                                     [10]

(c)  Compare the efficiency of the Carnot cycle to that of the Stirling cycle.                   [6]

4    a)  Show that  pVγ=A is the equation of state for an adiabatic transformation of an ideal gas if A is a constant.                                                                                                      [6]

b)  For the same final volume, show that a gas which experiences an adiabatic expansion has a final pressure, which is less than if the expansion is isothermal.                        [10]

c)   Explain what you have just shown in b).                                                                      [4]

5.          (a)        If  Maxwell’s distribution of speeds of molecules is given as:

dnv=4nπβ32v2e-βv2

- Find an expression for the most probable speed of the molecules	  [4]

- From this distribution show how you would compute the vrms of the molecules.                                                                                     [6]

- Express vrms as a function of pressure and density of the molecules of an ideal gas.                                                                                        [10]

6.    (a) State at least four  ways in which a substance may be stimulated to emit electromagnetic radiation                                                                                                                         [4]

(b) Define the following: Blackbody and  temperature gradient.                                  [4]

(c) Write down an expression for the heat transferred by radiation between two bodies of differing temperatures (Ta and Tb)                                                                                           [6]

(d)   Write down the fundamental Law of Heat Conduction defining all the quantities involved.                                                                                                                        [6]

END OF EXAMINATION